Walkthrough:  December 8, 2004

Warning!!!

GA-K8NS, GA-K8NS Pro and GA-K8NS Ultra 939 three entirely different motherboards… DO NOT GET THEM MIXED UP OR YOU WILL HAVE MAJOR PROBLEMS.

GA-K8NS Ultra 939

If the BIOS haven’t been updated, the POST screen will say “GA-K8NSNXP 939” or “GA-K8NSNXP Ultra” but the motherboard itself will say “K8NS Ultra 939”, the BIOS update will be “GA-K8NS Ultra 939”. Don’t worry; it is the right BIOS file.

BIOS settings for a single SATA hard drive, two SATA hard drives, two SATA hard drives with RAID-0, two SATA hard drives with RAID-1 on Silicon Image Controller:

(Notice: Improper settings will give you a blue screen 0x0000007B. This is not caused by the drivers this time, so try not to blame the Recovery CD)

- Advanced BIOS Features

- Hard Disk Boot Priority, press Enter and make sure that Bootable Add-in Cards is not the first one (sometimes it’s set to be the first one), press escape to go back to the previous screen

- First Boot Device should be set to [CDROM]

- Second Boot Device should be set to [Hard Disk]

- Third Boot Device should be set to [Floppy] and press escape to go back to the previous screen

- Integrated Peripherals

- IDE Function Setup, press Enter and everything should be disabled on that screen, press escape to go back to the previous screen

- For Endusers who have USB keyboards (check the invoice), make sure that USB Mouse and Keyboard Support are both enabled or they will complain that their keyboard is not working

- Serial-ATA 2 (Internal PHY) should be set to [Disabled]

- Onboard Serial ATA should be set to [Enabled] (of course)

- Serial ATA Function should be set to [RAID] not [BASE] (this is very important!)

- Press F10, save and exit (there is an F lock key on the keyboard is F10 is not working)

BIOS settings for a single SATA hard drive, two SATA hard drives, two SATA hard drives with RAID-0, two SATA hard drives with RAID-0 on nVIDIA RAID

At this point, you are now ready to format and reload the computer.

Warning!!!

The following steps with completely reload your operating system; you will lose all the information so please make sure to back up all your data before proceeding.